MA35D1 Core1: Real-time H.264 Video Decoding & AMP Synchronization Project

Technical Documentation & User Manual

Version: 1.0.0 | Date: 2026/05/12

Author: Nuvoton Development Team

1. Project Introduction

This project demonstrates a high-performance H.264 video decoding pipeline running on the Nuvoton MA35D1 Core1 (Cortex-A35). Unlike standard implementations, this system leverages a heterogeneous Asymmetric Multi-Processing (AMP) architecture.

Key Performance Targets:

- 720p (1280x720) Decoding: 70 FPS.

- 1080p (1920x1080) Decoding: 30 FPS.

- Peak USB Bandwidth: 16 MB/s (Bulk-OUT).

2. Hardware Architecture & Pin Configuration

2.1 Peripheral Interconnects

- USB Interface: HSUSBD (High-Speed USB Device) configured for Bulk transfer.

- Display Controller (DCUltra): Configured for 24-bit RGB888 output to a 1024x600 (or 1280x720) panel.

- UART Debug: UART16 (Baud: 115200, 8-N-1) for real-time logs and system statistics.

2.2 GPIO Assignments

| Function | GPIO Pins | Description |
| --- | --- | --- |
| LCD Data | GPH0-7, GPI8-15, GPC12-15, GPH12-15 | 24-bit RGB Data Bus |
| LCD Sync | GPG8 (VSYNC), GPG9 (HSYNC), GPG10 (CLK), GPK4 (DEN) | Timing Control |
| UART16 | GPK2 (RX), GPK3 (TX) | Debug Console |
| USB VBUS | PF15 | VBUS Validation |

3. Memory & Resource Management

3.1 Static Memory Mapping (SDRAM)

The system utilizes a strict memory partitioning strategy to ensure zero-copy handoff between the USB stack, VPU, and Display Controller.

- Core1 FreeRTOS Region (0x88000000): 32MB allocated for the RTOS kernel, task stacks, and the 2MB USB Circular Buffer.

- VC8000 Decoder Pool (0x8A800000): 32MB dedicated hardware memory for H.264 bitstream parsing and reference frame storage.

- DCUltra Frame Buffer (0x8C800000): 48MB region where the VPU Post-Processor (PP) writes decoded RGB888 frames directly for display.

- RPMSG Shared Memory (0x84000000): Shared communication buffer for Core0-Core1 synchronization.

3.2 Linux Integration (DTS Configuration)

The Core0 Linux kernel must reserve these regions to avoid memory corruption. Add the following nodes to your Device Tree:

reserved-memory {

core1_rtos: core1_rtos@88000000 {

reg = <0x0 0x88000000 0x0 0x2000000>;

no-map;

};

vc8k_buf: vc8k_buf@8a800000 {

reg = <0x0 0x8A800000 0x0 0x2000000>;

no-map;

};

display_buf: display_buf@8c800000 {

reg = <0x0 0x8C800000 0x0 0x3000000>;

no-map;

};

rpmsg_buf: rpmsg_buf@84000000 {

reg = <0x0 0x84000000 0x0 0x9000>;

no-map;

};

};

4. Software Implementation Details

4.1 USB Video Sink (usbd_video_sink.c)

- Transport Mode: USB Bulk-OUT on Endpoint 1.

- DMA Engine: Uses Dedicated DMA (DDMA) with descriptor chains (S_HSUSBD_DMA_DESC_T) to minimize CPU intervention.

- Data Flow: Data is received in 16KB chunks and pushed into a 2MB global circular buffer.

4.2 VC8000 Decoding Task (video_task.c)

- Post-Processor (PP): Configured to output VC8000_PP_F_RGB888.

- Zero-Copy Logic: The PP destination is set to VC8000_PP_OUT_DST_DISPLAY, allowing the decoder to write directly into the display memory pool.

- Performance Monitoring: Calculates FPS every 5 seconds using g_tmr2_ticks.

4.3 AMP & RPMSG Control (main.c)

- Dual Channels: Implements eptA->B for receiving commands and eptB->A for sending status.

- Command Interface: Supports interactive commands (Reset, Statistics, Heap check) via UART.

5. User Guide: How to Run

5.1 Host System Configuration (Windows)

- Drivers: Install WinUSB4NuVCOM.exe. Ensure the device is recognized as a WinUSB device in the Device Manager.

- Environment: Install Python 3.x and the pyusb library.

5.2 Preparing the Content

The decoder requires a raw H.264 Annex B bitstream. Use FFmpeg to transcode your video:

Bash

ffmpeg -i my_video.mp4 -vcodec libx264 -s 1280x720 -pix_fmt yuv420p -f h264 output.h264

5.3 Streaming Data

Run the provided Python script to start the stream:

Bash

python host/usb_vsink_play.py output.h264

6. Maintenance & Debugging

- Throughput Check: Press 's' in the UART console to view real-time AMP data throughput.

- Memory Health: Press 'h' to check FreeRTOS heap availability.

- System Reset: If the USB connection is lost, the system will attempt to re-initialize; alternatively, press 'r' to restart the AMP tasks manually.